9.26.05

Tuning for Ideal Pitch Field Linearity

by Christopher

The ideal pitch response from the Altermen antenna occurs when the pitch oscillator is tuned slightly "above" the LC resonance of the coiled antenna. This is tuning to the sweet spot for the maximum response of the pitch field. The antenna LC resonance is tuned slightly below the Pitch oscillator frequency to avoid frequency cross-over as your hand approaches.

The Altermen antenna coil spring has parasitic capacitance between the coil turns and capacitance within the environment. The overall spring stretch should be a maximum of 1/2" or less. IMHO the closely gathered spring coil turns resonate as part of a marriage with the tank circuit below 922 kHz. 9/22 is Michael Faraday's birthday, thoughts from the grave.

The pitch oscillator should have the highest Q while the antenna response I believe is less so.

‘--------------------------

My Speculation: Moving your hand closer to the Altermen Antenna, hand capacitance #1 will lower the Altermen Antenna resonant frequency which begins to de-tune it from being part of the Pitch oscillator harmony. This detuning reduces the load on the pitch oscillator slightly.

Less load the variable Pitch oscillator Xc capacitance reactance which lowers the LC tank capacitance #2.

and this counters drives the variable oscillators frequency higher.

‘---------------------------

The highest notes next to the Pitch antenna would be most affected. This is why with proper "Altermen" LC tuning, compression of the higher notes cannot occur next to the antenna. It is at this point next to the antenna that the pitch field energy has been reduced to a minimum by the off tuning of the antenna LC combo circuitry.

This effect occurs using either tube/valve oscillators or BJT transistors. Also no inline chokes are used as the coil antenna is an inductive coil with a direct connection to the Pitch oscillator tank circuit. I did not experience theremin non-linearity until I purchased several EtherWave Standards years later. Properly tuned they can be very good also.

I originally saw this balancing effect while studying the LC character of a coiled antenna back in 2004. My tests made it obvious that something unusual was happening. Using a 12" to 24" pitch field it is still linear.

"The main phenomenon found in the Altermen antenna is that the musical notes found in the pitch field will spread out to match a piano keyboard perfectly note for note, how would Nature have known this?

Another phenomenon, depending on where the connecting wire is secured along the row of coils it will counter balance against drift caused by the environment. In other words to far to one side the theremin pitch will rise as the temperature rises, to far to the opposite side the pitch will lower as the temperature rises."

Christopher