Phoenix RF Oscillator Theory of Operation

hFE is another expression of Forward Current Gain or Beta, it is current gain. From something I learned long ago biasing the transistor properly takes this out of play. If I have learned anything over the years it is biasing and my approach is the standard and good.

It is my belief due to the thermal sensitivity of a bipolar transistor that untouched at room temperature and identifying two that have the same “rough” measurements of hFE will have similar behavior to resist thermal drift, much needed in theremin design. Vacuum tube theremin’s have much less drift.

My own follow up theory to this is you can minimize thermal drift by balancing or shifting the current in each transistor. This is done with Pot-3 1k on the L2 side of the board. The L1 circuit uses the resistor R9 470 ohms to keep in balance with Pot-3 1k set half way.

I observed long ago while experimenting I could shift the direction of drift; if the Pitch would rise with temperature rise I could adjust Pot-3 and make the Pitch drift go lower or in the opposite direction. Somewhere in-between is balance. Even if not perfect adjusting the Pot-3 to one side or the other will be much better than no adjustment.

|  | The use of the MPSA42-AP transistor has special qualities discovered by experimentation. In theremin design to get a more natural sound my thought was let’s make the RF section more sluggish, this idea comes from my vacuum tube design in how it had a better droop at the bottom of the wave shape. The skew on top is the even harmonics. This is the throatier sound; transistors are too dam snappy. |
| --- | --- |
| That is why with the D1 1N914 diode uses a micro current. The transistors also run at very low current but this is to limit the intensity of the magnetic field around the inductors L1 & L2, so they do not interact. An original idea was to pick up the magnetic fields of both inductors using a third inductor L3. It worked outstanding, isolating the L1 & L2 from one another with no need to add extra buffering components. This approach by placing L3 closer to L1 increases the amplitude through D1, 0v-750mv. It is here I adjust the sine wave that works best for the sound I want. The follow up original opto-Isolator with the transformer does the final wave shaping for an ideal theremin voice. |  |

For the Volume Control we want L1 & L2 to interact. There is a switch S1 for this, the right wave shape creates the Pulse Width Modulation that works for Volume Control.

Read about the Becker Electrodeum, another remarkable discovery for Pitch linearity.

Important: If you used my values in the parts and swapped the MPSA42-AP transistor with a 2N3909 the Pot-4 sweep range to find Zero-Beat will increase by more than double. The 2N3909 has much more junction capacitance prone to thermal drift. The reason this method of tuning works is by varying the voltage across the transistor junction, this shifts the junction capacitance, like a varactor diode. Adding a 470 ohm or lower value of resistor at R25 in parallel with Pot-4 is how you narrow the Zero-Beat tuning range. When experimenting I like a large sweep range, Thereminist want just enough for easy tuning.

The Phoenix works at 900 kHz and an ideal way to monitor what is going on is to tune an “analog” old school AM Radio till you hear the heterodyne tone around 900 kHz. It is a beautiful first experience. This sound & discovery is what led Lev Sergeyevich Termen to development the theremin. This method also allows you to hear at what approximate frequency the two oscillators are at. You will hear a blank spot in the radio static. To get the heterodyne tone both frequencies must be tuned to be on top of one another.

Christopher – oldtemecula  4.15.20                                       www.Hwy79.com