Phoenix 2017   6.12.17

The 470uf caps were changed to 1000 uf, the pitch board use one at the regulator and one at the 3 x 3 filter with the 100 ohm resistor. The volume control uses one at C9 to smooth the PWM output to drive the Vactrol.

Phoenix 2017   6.11.17

2N3904 NPN transistors were place back on the Mouser list so it is complete.

When using these for the oscillators, a 1K pot is used for tuning at the Pot-4 connections.

You may need to add the R25-470R resistor to dampen the tuning response.

MPS-A42 NPN transistors are preferred for the oscillators on the 3 x 3 section.

When using these, a 2K pot is used for tuning at the Pot-4 connections, no R25.

This pot difference is the first indicator of transistor junctions with better thermal drift resistance.

In proper circuit design the hFE characteristic of a transistor is not in play. I still like to use two transistors in the theremin oscillator section that have matching hFE.

hFE of a transistor is the current gain or amplification factor of a transistor. hFE (which is also referred to as β) is the factor by which the base current is amplified to produce the amplified current of the transistor.

My Harmonic Exciter breakout board for capturing Clara’s harmonically rich sound was developed from using the Phoenix 3” x 5.2” section, they are almost identical. Overdriving the audio transformer does the trick.

Exciter Schematic Here

RF signal mixing is done by taking a signal off both ungrounded IF transformer cans at the ABCD terminals and passing them through the mixing diode D2 on the 3 x 3 section. Pot-5 100k has some control over forward biasing D2 and wave shaping.

This is the result of using the 100R/1000uf filter enhancement.  Mouser List?

R31-100 & C29-1000 uf