7.21.20

Phoenix Thermal Drift

Pitch side must be 900 kHz to resonate with the Electrodeum pitch antenna. Volume side can be any freq that does not interfere in the Pitch sound. Thermal drift comes from PN junction capacitance which changes in how warm they are or the voltage across them.

The Phoenix method of drift control shifts the voltage across the PN junction. My theory is the transistor bias values setup the transistor as a current source. Changing the transistor load (Pot-3 1k) shifts the voltage across this current source. The PN junction capacitance changes with temperature or the voltage across it. This is why varying the emitter resistor value can be used for fine tuning the oscillator frequency.

My vacuum tube theremin design had little if any thermal drift as they have no PN junctions yet there was heat changes around the large coils. My experiments with tubes is what revealed where most solid-state drift comes from.

I have avoided using variable capacitors until this last theremin build as I always thought being mechanical they would also add to drift.

Theremin design uses principles so sensitive they reveal phenomenon about components an electronic designer might never take into consideration. Two identical transistors and part numbers could be used and work perfectly together yet when choosing two other transistors from the same batch could drift all over the place. This is why I often say using computer modeling with theremin oscillators will most likely miss the mark in operation and reliability.

I used the MPSA42 Transistors to test if the 300 volt junctions might have less PN junction influence. This still needs more study but stayed with them as they created a wave shape that mixed for a slightly better sound. I did try 1000 volt transistors but they would not oscillate with my favored oscillator circuit design.

* A test that needs to be done is find which is more stable? Vary the emitter resistance or use a low value variable panel mount capacitor and leave the PN junction voltage fixed.

The EWS drift is no better than my approach.